ARRL Technical Information Service

Morse Transmission Timing Standard

File: code-std.txt

Updated: 12-13-96

Author: Michael Tracy, KC1SX (email: tis@arrl.org)

Reprinted from: April 1990 QEX "A Standard for Morse Timing Using the

Farnsworth Technique"

Copyright 1990 by The American Radio Relay League, Inc.

All rights reserved.

Prepared as a membership service by the American Radio Relay League, Inc.,

Technical Information Service.

For your convenience, you may reproduce this information,

electronically or on paper, and distribute it to anyone who needs

it, provided that you reproduce it in its entirety and do so free

of charge. Please note that you must reproduce the information as

it appears in the original, including the League's copyright

notice.

If you have any questions concerning the reproduction or

distribution of this material, please contact Michael Tracy,

American Radio Relay League, 225 Main St., Newington, CT 06111

(email: tis@arrl.org).

-----------------------------------------------------------------

A Standard for Morse Timing Using the Farnsworth Technique

by Jon Bloom, KE3Z, ARRL Laboratory

In recent years there has been a renewed interest in Farnsworth timing

of Morse transmissions.  Farnsworth timing is defined as sending the

characters at a faster speed than the words.  For example, sending the

characters at 20 WPM but adding enough time between them to slow

down the rate to 10 WPM.

The problem in learning Morse is that at speeds above a few WPM,

Morse is most easily read by ear when the characters are recognized

rhythmically rather than by counting the dots and dashes.  But the

person just learning Morse starts at very slow speeds, where counting

is easier than recognizing the slow rhythm of the characters.  So in

order to increase their ability to read Morse above a few words per

minute, students are forced to shift from the counting mode to the

rhythm recognition mode.  This probably the cause of the oft-

mentioned "13-WPM barrier."

The idea behind the Farnsworth method is to eliminate the counting

phase by sending the characters at a speed at which rhythm recognition

is easy and counting is not.  This forces the student to learn the

rhythms.  Initially, the rate of transmission is slowed (by the addition of

time between characters) to allow the student to gradually build the skill

at recognizing and writing the received text.  The process by which the

student recognizes the characters is never changed; he just gets better

(faster) at doing so.

Recently, ARRL, finished converting all of its Morse material to

Farnsworth timing.  ARRL, is using a standard of sending transmission

at an 18-WPM character rate. (Of course, at 18 WPM and faster

speeds, ARRL transmission revert to standard timing, since no extra

time has to be inserted.)  This standard applies to all code practice and

test tapes, and to W1AW transmissions.

In implementing Morse generation here at ARRL. We ran across a

problem:  There is no standard for Farnsworth timing.  In fact, we

couldn't find any definitive specification for how Farnsworth timing is

calculated.  It's fine to say that you're going to transmit, for example, a

10 WPM text using 18-WPM characters, but exactly how much time

needs to be added to the transmission, and where?  The ARRL code

tapes and W1AW transmissions are generated by computers, and you

need a specific answer to that question to write the computer program.

Thus, the ARRL, has settled on a standard character speed of 18

WPM, this is not inherent in the idea of Farnsworth timing - any speed

can be used.

From QEX, April, 1990 Page 8/9

The ARRL Morse Transmission Timing Standard

1. General

This standard is motivated by recent changes in the systems

used to generate Morse text for ARRL.

1.1 Scope

This standard defines the timing parameters used for all ARRL

Morse training materials, including code-practice tapes, code tests and

W1AW Morse transmissions.

2. Timing

At speeds of 18 WPM and above, standard timing specified in

2.1 will be used.  At speeds below 18 WPM, Farnsworth timing

specified in 2.2 will be used.

2.1 Standard timing

Standard timing is as follows:

The period of a single dot is one unit, measure in seconds.

A dash is a period of three units.

A period of one unit separates each element (dot or dash) within a

character.

A period of three units separates each character within a word.  A

period of seven units separates each word.

For purposes of specifying code speed, the "PARIS" 50-unit

standard is used. (1) From that standard, the following relationship is

derived:

u = (1.2)/c

where:

u = period of one unit, in seconds

c = speed of transmission, in words per minute (WPM)

2.2  Farnsworth timing

At speeds below 18 WP{M, characters are sent using 18-WPM,

characters are sent using 18-WPM timing, but with extra delay added

between characters and words to produce an overall lower speed.

Speeds are specified as s/c, where s is the overall transmission

sped and c is character speed. For example , a 5-WPM transmission

sent with 18-WPM characters is specified as 5/18 speed.

The character timing used is as specified in 2.1 (above), using

the unit, dot and dash periods., as well as the one-unit inter element

spacing.  The adjustment to a lower speed is made by adding delay

between characters and words.  The added delays are constant for a

given Farnsworth speed and will maintain the 3/7 ration of character

space to word space.

The added delays are calculated as follows:

Ta = (60c - 37.2s)/sc

Tc = (3Ta)/19

Tw = (7Ta)/19

where:

Ta = Total delay to add to the characters (31 units) of a

standard 50 - unit word, in seconds

Tc = period between characters, in seconds

Tw = period between words, in seconds

Reference

(1)  Hale, Bruce S., et al, The 1989 ARRL Handbook for the Radio

Amateur, Newington, CT, ARRL, 1988, p 19-4

Appendix A - Derivation of Timing Equations

A. 1 Unit period

The unit period, u, is derived from the "PARIS" 50 - unit

standard as follows:

s words of 50 - units each transmitted in the space of one

minute are, by definition, being transmitted at s words per minute.

Thus, units are occurring at 50s unites per minute.  The equation is:

r = 50s            where r is the rate in units/minute

To convert to units/second:

r (units/minute) x (1 Min/60 Sec) = (r/60) = (50s/60) = (5s/6)

the reciprocal gives u, the period of a unit in seconds:

u = (6/5s) = (1.2/s)

A.  2 Farnsworth Timing Delays

The total delay added to each 50 - unit word transmitted is the

difference between the time it takes to send the word using standard

timing at speed s (the overall speed) and the time it takes to send just

the characters at speed c (the character speed).  The time it takes to

send a 50 - unit word at speed s is, by definition:

T50 = 50 x (1.2/s) seconds

A  standard 5 - letter, 50-unit word contains 31 units of element and

interelement spacing (that is everything exclusive of intercharacter and

interword spacing).  The time it takes to send 31 units at speed c is:

T31 = 31 x (1.2/c) = (37.2/c) seconds

The difference between these two times at a given Farnsworth

(s/c) speed is therefore:

Ta = (60/s) - (37.2/c)  where s and c are as defined in paragraph

2.2

or, by algebra:

Ta = (60c - 37.2s)/sc

In the transmitted word, this delay is divided among four intercharacter

spaces, each Tc long, and one interword space Tw long, representing

19 total units (4 x 3 + 7 = 19).  This gives the relationships for the

division of Ta into these delays:

Tc = (3Ta)/ 19  and  Tw = (7Ta)/19

---------------------------------END------------------------------

Note: The Farnsworth timing came from the late Donald R. "Russ"

Farnsworth, W6TTB, who in the late 1950s asked Bart Bartlett, W6OWP,

to help him prepare some tapes for a code course he had developed.

Farnsworth's unique method of instruction was to maintain the code

speed at a constant 13 WPM throughout the course, but starting with

simple text and increasing the complexity of the text material as the

course progressed. (excerpt from the April 1988 QST Correspondence

column). He later went on to create Epsilon Records which sold a CW

learning system on long playing records.

The text of this Standard has been entered as a Standard ASCII text

file, without using the text formating that was available in the original

QEX article.  Where subscripts have been used in the original they are

replaced by capital letters followed by a lower case letter.  Where

division is performed the leading numerator is shown in brackets "("

and ")" and the denominator following the "/".

Notes by Larry Kayser, VA3LK / WA3ZIA

*EOF