How you can see my location on the web:

Go to  http://map.findu.com/n6rdk-9

How it works:

There are many steps involved as a cooperative effort to enable web display of APRS.  First, I transmit my position with the beacon box (1). Then, a digipeater in one of the various places, such as a mountain, receives my signal and retransmits it throughout a wide region (2).  Quite possibly, another digipeater continues this transmission further.  This can happen for several hops and can move the message hundreds of miles in seconds. One of the ground stations, Greg Noneman WB6ZSU-3, receives the message.   His computer in Irvine is connected to the Internet (3).  He forwards everything he hears in the LA basin to a server located in Florida (or wherever – it doesn’t matter) (4).  Steve Dimse K4HG operates this server and has created special software to monitor these data and make the data available via the web by using his APRServe program.  He also operates a Java interface for realtime display using a browser. However, by using port 8000, his program will display the maps showing the selected user’s position on your own browser (5).